ROHM Switching Regulator Solutions

Evaluation Board:

Step-down Switching Regulator

With Built-in Power MOSFET

No.0000000015

BD9G101G (5V | 0.5A Output)

●Introduction

This application note will provide the steps necessary to operate and evaluate ROHM’s step-down switching regulator using the BD9G101G evaluation boards. Board layout recommendations, operation procedures, application data and build of materials are provided.

●Description

This evaluation board has been developed for ROHM’s step-down switching regulator customers evaluating BD9G101G. While accepting a wide power supply of 6-42V, a step down output of 1V-29V can be produced. The IC has an internal 800mohm Nch MOSFET (1A max) and a fixed operating frequency range of 1.5MHz. A Soft Start circuit prevents in-rush current during startup along with UVLO (low voltage error prevention circuit), OCP (over-current protection) and TSD (thermal shutdown detection) protection circuits. An EN pin allows for simple ON/OFF control of the IC to reduce standby current consumption.

●Applications

Industrial distributed power applications

Automotive and battery powered equipment

●Evaluation Board Operating Limits and Absolute Maximum Ratings

| Parameter |  | Symbol | Limit |  |  | Unit | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MIN | TYP | MAX |  |  |
| Supply Voltage |  |  |  |  |  |  |  |
|  | BD9G1010G | VCC | 6 | - | 42 | V |  |
| Output Voltage / Current |  |  |  |  |  |  |  |
|  | BD9G101G | VOUT | - | 5 | - | V | *Set by R1 and R2 |
|  | BD9G101G | IOUT | - | 0.5 | - | A |  |

●Evaluation Board

Below is the evaluation board with the BD9G101G. Component selection and board layout guidelines are provided in the BD9G101G datasheet.

VOUT

Vout

GND

GND

VCC

BD9673EFJ Eval Board

ENABLE

H: Active (ICOFF)

L: Inactive

(ICON)

EN

BD9G101G Eval Board (Back)

BD9G101G Eval Board (Front)

●Evaluation Board Schematic

Below is the evaluation board schematic for BD9G101G.

10uH

●Evaluation Board I/O

Below is the reference application circuit that shows the inputs (Vcc and EN) and the output (Vout @ 5V/0.5A)

L1: 10uH

●Evaluation Board Operation Procedures

Below is the procedure to operate the evaluation board

- Connect GND to a GND pin on the evaluation board

- Connect Vcc to the VCC_pin. This will provide Vcc to the Vcc pin of the IC

    - Note: EN pin is pulled HIGH as default. EN needs to be set to LOW for IC operation.

- Output voltage can be measured from the VOUT_pin on the evaluation board. Output current can be measured with a proper load at VOUT.

●Reference Graphs Application Data for BD9G101G

Below graphs show efficiency, frequency response and load characteristics of the BD9G101G eval board.

- Note: Feed-forward capacitor Csp is not populated on the BD9G101G evaluation board. The capacitor is recommended for designs requiring improved phase margin and transient response.

●Evaluation Board Layout Guidelines

Below are the guidelines that have been followed and recommended for BD9G101G designs.

●Evaluation Board BOM

Below is a table with the build of materials. Part numbers and supplier references are provided.

| Item | Qty | Ref | Description | Manufacturer | Part Number | Digikey P/N |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | CIN1 | CAP CER 4.7UF 50V Y5V 1206 | Murata | GRM31CF51H475ZA01L | 490-1828-1-ND |
| 2 | 1 | CIN2 | DIODE TVS 30V 400W UNI 5% SMA | Littlefuse | SMAJ30A | SMAJ30ALFTR-ND |
| 3 | 1 | CO2 | CAP CER 10UF 10V 10% X5R 1206 | Murata | GRM31CR61A106KA01L | 490-1820-2-ND |
| 4 | 1 | CBST | CAP CER 0.015UF 50V 10% X7R 0603 | Murata | GRM188R71H153KA01D | 490-1514-1-ND |
| 5 | 1 | D1 | DIODE SCHOTTKY 60V 2A PMDU | ROHM | RB060M-60TR | RB060M-60CT-ND |
| 6 | 1 | SW1 | SWITCH SLIDE SPDT 30V.2A PC MNT | E-Switch | EG1218 | EG1903-ND |
| 7 | 1 | R1 | RES 68.0K OHM 1/8W 1% 0805 SMD | Yageo | RC0805FR-0768KL | 311-68.0KCRCT-ND |
| 8 | 1 | R2 | RES 12.0K OHM 1/8W 1% 0805 SMD | Yageo | RC0805FR-0712KL | 311-12.0KCRCT-ND |
| 9 | 1 | LX | INDUCT 10uH SOD-106 | TOKO | DEM4518C 1235AS-H-100M | - |
| 10 | 1 | U1 | 5V, 0.5A Step-down SW Reg w/ FET | ROHM | BD9G101G | - |